Template    Deadline: 5 November 2025

Deliverable D3.2: Policy and Governance Evaluation Report (Due 01 March 2026)
Partner / Living Lab: Nappe Charf El Akab
Country: Morocco
Contact person(s): Pr. Abdes Samed Bernoussi
Email: abernoussi@uae.ac.ma

1. General Description of the Living Lab

(Max ½ page)

- Short overview of the site (location, area, climate, cropping systems, water sources).

The Charf El Akab aquifer, located 20 km south of Tangier and covering about 17 km², is bordered by low permeability Tangier pelites and includes two main layers an upper and a lower aquifer. The lower layer, which has better hydrodynamic properties, is exploited by the National Office for Electricity and Drinking Water (ONEE). The area has a Mediterranean climate with agricultural and forestry land use. Recharge is mainly artificial, through infiltration basins and injection wells supplied by the Mharhar River and, more recently, by treated water from the El Hachef plant.

Main local socio-economic actors include groundwater dependent farmers and agricultural cooperatives in the Charf El Akab area, using wells to irrigate Mediterranean crops and sustain rural incomes. (ONEE) and the Regional Multiservice Company of Tangier–Tetouan–Al Hoceima (SRM TTH) pump from the aquifer to secure domestic and industrial supply for the municipalities of Tangier and Asilah. These uses are regulated and coordinated by the Loukkos Hydraulic Basin Agency (ABHL).

- Main objectives of the Living Lab within SAVE Water.

The Charf El Akab Living Lab aims to develop and test innovative solutions for sustainable groundwater management. Its main goals includle improving artificial recharge, implementing smart monitoring tools, protecting water quality, and reinforcing participatory governance. By involving local stakeholders, it promotes a more resilient and water-smart use of the aquifer.

2. Stakeholders Involved in Water Management

(Add rows as needed. Please include a diverse range of stakeholder types such as: government institutions, research organizations, farmer associations, NGOs, private sector actors, and local or regional authorities. The aim is to capture the full governance landscape of your Living Lab.)

- Regional Multiservice Company of the Tangier-Tetouan-Al Hoceima Region (SRM TTH)

- Loukkos Hydraulic Basin Agency (ABHL)

- Directorate of Water Research and Planning - Ministry of Equipment and Water (DRPE)

| Stakeholder / Institution | Type (Gov./Farmer Org./NGO/Private/Research) | Main Role in Water Management | Level (Local/Regional/National) |
| --- | --- | --- | --- |
| ABHL SRM TTH | GOV GOV | - Preservation, management, and planning of aquifer use - extraction and withdrawal from the aquifer | - Tétouan, Tangier-Tetouan-Al Hoceima Region, Morocco - Tangier, Tangier-Tetouan-Al Hoceima Region, Morocco |

3. Governance Framework

(1 page max)

- Main laws, regulations, or policies governing water use and allocation.

- Law 36 15 on water states that all surface and groundwater are part of the public hydraulic domain, and that major abstractions, drilling and recharge works need prior authorization or concession framed by basin level PDAIRE plans.

- The Loukkos Hydraulic Basin Agency (ABHL) prepares the PDAIRE, delivers water-use permits, validates new or increased groundwater pumping, and signs participatory aquifer contracts defining volumes, protection zones and monitoring duties for aquifers such as Charf El Akab.

- ABHL works with the Regional Multiservice Company of Tangier–Tetouan–Al Hoceima (SRM-TTH), which runs drinking-water infrastructure and must abstract groundwater in accordance with ABHL licenses, operating rules and emergency-supply arrangements.

- Responsible institutions and their coordination mechanisms.

- Loukkos Hydraulic Basin Agency (ABHL)

- Control, preservation, and sustainable management of aquifer water exploitation in consultation with the various users within the framework of the aquifer agreement and according to the provisions of Law 36/15 on water

- Regional Multiservice Company of the Tangier-Tetouan-Al Hoceima Region (SRM TTH)

- Exploitation of aquifer water according to ABHL instructions

- Any decentralization, water-user associations, or basin authorities.

- Monitoring, preservation, and sustainable management committees of the aquifer designated within the framework of the aquifer contract

- Technical committee of the basin council

- Decentralization relies on basin agencies like ABHL, which are autonomous bodies; basin councils and technical committees gather administrations, municipalities and users to review plans, aquifer contracts and major permits.

- Participatory aquifer contracts set up local committees (ABHL, authorities, municipalities, users) that monitor implementation, suggest adjustments and help resolve conflicts.

- A basin “water police” inspects wells and meters and can sanction illegal or non‑compliant pumping through fines, closure or regularisation orders, and, if needed, court action.

- Existing monitoring or licensing systems for groundwater / surface water.

-  Groundwater monitoring network through piezometers

-   numerical model for aquifer management

-   For the Charf El Akab aquifer, ABHL runs a monitoring network (piezometers, observation wells and quality sampling points) that feeds a numerical model to simulate recharge, estimate safe yields and define abstraction limits.

-   SRM-TTH and other major users install meters and report pumped volumes, and these data, combined with monitoring results, allow ABHL to update permits, adjust artificial recharge and react when thresholds for groundwater level or quality are exceeded.

4. Participation and Decision-Making Processes

- How are local stakeholders (farmers, municipalities, etc.) involved?

- Through specific aquifer contract agreements and participation in decision-making within the technical committee of the basin council.

- Are there formal participation structures (committees, workshops, co-creation meetings)?

- Technical committees of the aquifer contract and the basin council.

- How are gender and social inclusion considered?

- Through awareness-raising events, communication, and sharing updates regarding the aquifer within the framework of the aquifer contract or basin council, or on the official ABHL web pages.

5. Challenges and Needs

- Key governance gaps or obstacles (coordination, transparency, enforcement, etc.)

- Comprehensive surveys of extractors from the aquifers

- Illegal withdrawals from the aquifer

- Lack of awareness of the aquifer's importance for future generations

- A modern artificial recharge system

- Data gaps or institutional barriers.

- Climatological station in the aquifer to monitor precipitation and evaporation

- Smart meters for withdrawals

- Radar satellite images to assess withdrawals

- Monitoring is constrained by the lack of a dedicated climate station on the aquifer, the uneven installation of smart meters, and only limited use of satellite tools to estimate withdrawals, which weakens water balance assessments.

- In addition, data from piezometers, utilities and agricultural services are not always harmonised or shared in real time, creating fragmented information, delays, and too little common evidence for joint decision making.

- Main needs for policy support or capacity-building.

- Strengthening and equipping water police teams to control withdrawals from the aquifer

6. Existing Documents / Data Sources

- Please list or attach any relevant national or local reports, regulations, or data sources (PDF / link / title).

- Geophysical study for the accuracy of the aquifer geometry (PDF)

- Aquifer modeling study (PDF)

7. Contact for Follow-Up

Name(s), role(s), and best contact channel for further discussion or clarification.

Anass BOuKHOLLA
Chef de Service Suivi Évaluation des ressources en eau à l'ABHL
Email : anassboukholla@gmail.com